# Manta AUV: Estimator Log

Date: 2026-10-04 (Day 1)

Project: 6-DOF manta-ray AUV, 8 BlueROV2-Heavy (T200) thrusters, sole sensor VN200 GPS-INS (no DVL, no depth sensor). Notation: Fossen (2011).

Scope decided today: estimator first. Plant model deferred to a later stage. Work on main; user makes all commits.

# 1. Session goal

Set up the research direction, agree on the equations to be checked, settle the flooded-cavity modelling question, review the Lambert et al. Part 1 paper, and start the daily log.

# 2. Decisions

- Repository: empty at start (no commits, no files, no remote branches). Work happens on main; the user commits.

- Order of work: estimator first; plant model later.

- Code: no project code is written until the user approves. This log is produced by a helper script kept outside the repo (exception approved for the log only).

- Novel contribution: the user does not want to pursue the cavity-identification contribution suggested earlier. Dropped.

- CFD: own CFD of the custom vehicle in ANSYS. Vehicle and cavity dimensions to follow from the user.

- Log: one Word file per working day in log/, named YYYY-MM-DD_estimator_log.docx.

# 3. Flooded-cavity discussion (outcome)

Vehicle has a carbon-fibre outer skin with an open bottom, forming a flooded cavity between skin and internal frame.

- My first model was wrong. I proposed a through-flow "ram" force with water impinging on the roof. That needs an outlet. With a solid roof and no vents, net flux through the opening is zero, so the cavity water moves with the vehicle (user correction, accepted).

- Result: the cavity water acts as extra mass, not a new force. Roof pressure increase during heave acceleration a is about ρ·a·h, smooth, lasting only while accelerating. No impact.

- Up/down asymmetry belongs in damping (bluff open mouth leading on descent), not in the inertia. Potential-flow added mass is a quadratic form and cannot depend on direction; Lambert et al. Part 1 Fig. 17 agrees.

- Open points carried forward:

  - Entrained fraction κ: 1 for a deep narrow cavity, possibly below 1 for a wide shallow cavity. To be measured, not assumed.

  - Every gap, vent, drain or thruster opening is an outlet and lowers κ. Opening audit needed.

  - Double-counting risk: if the ANSYS added mass is computed with the cavity flooded and open, it already contains the cavity water. Do not add ρV_c on top.

  - Rotation: inviscid water in a non-axisymmetric cavity carries less rotational inertia than rigid. Rigid and inviscid values bound it. Hand estimate (assumed h = 0.1 m, ν = 1e-6 m²/s, Ω = 0.5 rad/s): Ekman spin-up time ≈ 140 s, so for manoeuvres of seconds the inviscid value is the relevant one. Needs real dimensions.

  - Trapped air under the roof would cause sloshing and depth-dependent buoyancy. Venting the top skin removes this but creates an outlet (see κ).

- Suspicious numbers to confirm: cavity 30 L (≈31 kg water) vs vehicle 12 kg. (12+31)/12 ≈ 3.6, not "two to three", unless external added mass is already in the base.

# 4. Paper reviewed

Lambert, Brizzolara, Woolsey, "Depth dependent added mass computations using impulse motion simulations for shallowly submerged vehicles, Part 1: Accelerating from rest", Applied Ocean Research 161 (2025) 104656.

- Method: smooth cosine acceleration ramp from rest (≈1 ms), least-squares fit F = M_A·ν̇ + D·ν. FOAM-Extend / Naval Hydro Pack, whole-mesh motion, inviscid and viscous runs, 2 m 6:1 prolate spheroid, diagonal terms only.

- Findings: added mass independent of acceleration amplitude (0.02%); inviscid and viscous agree for short ramps; near-surface reduction ≈12.5% sway and >20% surge at 0.75 D; coupled motions superpose; ± heave identical; kinetic-energy method within ≈1% (3.3% surge).

- My hand check: Lamb k-factors for 6:1 spheroid, k₁ ≈ 0.045, k₂ ≈ 0.917; with ρ = 1000 kg/m³ these give ≈5.26 kg (surge) and ≈106.7 kg (sway/heave), matching the paper's reference lines.

- Flag: abstract says within 0.5% of Lamb; surge converges to ≈5.43–5.46 kg vs 5.26 kg (≈3–4% high). Damping values from 1 ms ramps are not usable for manoeuvring.

- Relevance: the VN200 needs the antenna above water for GPS, so the vehicle surfaces, which is where added mass changes most. Deep down a constant M_A is acceptable.

Part 2 ("Accelerating from steady forward velocity", DOI 10.1016/j.apor.2025.104657) located but not readable: ScienceDirect is blocked from this environment. User to upload the PDF.

# 5. Equation status

Equations listed in the session checklist (kinematics, kinetics, Coriolis, added mass, restoring, EKF, consistency, controllers, fixed-point). Status of each:

| Item | Status | Note |
| --- | --- | --- |
| Kinematics J(η), Euler-rate T(Θ) | Unverified | Check against Fossen (2011); singular at θ = ±90° |
| M_RB, C_RB, C_A | Unverified | Check signs against Fossen; test νᵀCν = 0 |
| Restoring g(η) | Unverified | Check against Fossen; energy test |
| Planar 3-DOF M, C | Unverified | Check against Fossen |
| Strapdown error model (15-state) | Unverified | Derivation due next session |
| EKF predict/update, Joseph form | Standard | Textbook form |
| NEES / NIS χ² tests | Standard | Bar-Shalom formulation |
| Cavity ram-force model | Wrong | Withdrawn; replaced by entrained mass |
| Entrained cavity mass ρV_c (sealed) | Accepted | Upper bound; κ to be measured |
| Inviscid ellipsoid-cavity inertia | Unverified | From memory; check against Lamb |

# 6. Results run

None. No code was written or run today.

# 7. Inputs still needed from the user

- Vehicle and cavity dimensions (cavity height, opening size and location).

- List of every opening in the skin (vents, gaps, drains, thruster openings).

- Confirmation of 12 kg vehicle mass and 30 L cavity volume.

- VN200 datasheet values: gyro and accelerometer noise density, bias instability; confirm whether it has a barometer.

- ANSYS added-mass results when available (cavity flooded and open).

- PDF of Lambert et al. Part 2.

# 8. Next session

Estimator derivation only, no code: state vector, process model with IMU inputs, thrust-model velocity aid, VN200 measurement model, observability of the GPS-denied case. Written into the log for line-by-line checking.